ASSIGNMENT 4

| Date | 15 Nov 22 |
| --- | --- |
| Name | Mohammed sameer s |
| Team ID | PNT2022TMID38287 |
| Project Name | IOT Based Smart solution for railways |

QUESTION :

Write code and connection in wovki for ultrasonic sensor. Whenever distance is less than 100 cms send “alert” to IBM cloud and display in device recent events

CODE :

#include <WiFi.h>//library for wifi

#include <PubSubClient.h>//library for MQtt

WiFiClient wifiClient;

String data3;

#define ORG "m5c3wo"

#define DEVICE_TYPE "sameer"

#define DEVICE_ID "assignment_4"

#define TOKEN "12345678"

#define speed 0.034

#define led 14

char server[] = ORG ".messaging.internetofthings.ibmcloud.com";

char publishTopic[] = "iot-2/evt/sameer/fmt/json";

char topic[] = "iot-2/cmd/event_1/fmt/String";

char authMethod[] = "use-token-auth";

char token[] = TOKEN;

char clientId[] = "d:" ORG ":" DEVICE_TYPE ":" DEVICE_ID;

PubSubClient client(server, 1883, wifiClient);

const int trigpin=5;

const int echopin=18;

String command;

String data="";

long duration;

float dist;

void setup()

{

Serial.begin(115200);

pinMode(led, OUTPUT);

pinMode(trigpin, OUTPUT);

pinMode(echopin, INPUT);

wifiConnect();

mqttConnect();

}

void loop(){

bool isNearby = dist < 100;

digitalWrite(led, isNearby);

publishData();

delay(500);

if (!client.loop()){

mqttConnect();

}

}

void wifiConnect(){

Serial.print("Connecting to "); Serial.print("Wifi");

WiFi.begin("Wokwi-GUEST", "", 6);

while (WiFi.status() != WL_CONNECTED){

delay(500);

Serial.print(".");

}

Serial.print("WiFi connected, IP address: "); Serial.println(WiFi.localIP());

}

void mqttConnect(){

if (!client.connected()){

Serial.print("Reconnecting MQTT client to "); Serial.println(server);

while (!client.connect(clientId, authMethod, token)){

Serial.print(".");

delay(500);

}

initManagedDevice();

Serial.println();

}

}

void initManagedDevice() {

if (client.subscribe(topic)){

// Serial.println(client.subscribe(topic));

Serial.println("IBM subscribe to cmd OK");

}

else{

Serial.println("subscribe to cmd FAILED");

}

}

void publishData(){

digitalWrite(trigpin,LOW);

digitalWrite(trigpin,HIGH);

delayMicroseconds(10);

digitalWrite(trigpin,LOW);

duration=pulseIn(echopin,HIGH);

dist=duration*speed/2;

if(dist<100){

String payload = "{\"Alert Distance\":";

payload += dist;

payload += "}";

Serial.print("\n");

Serial.print("Sending payload: ");

Serial.println(payload);

if (client.publish(publishTopic, (char*) payload.c_str())){

Serial.println("Publish OK");

}

}

if(dist>100){

String payload = "{\"Distance\":";

payload += dist;

payload += "}";

Serial.print("\n");

Serial.print("Sending payload: ");

Serial.println(payload);

if(client.publish(publishTopic, (char*) payload.c_str())){

Serial.println("Publish OK");

}

else{

Serial.println("Publish FAILED");

}

}

}

OUTPUT :

- When Distance greater than 100 cm

IBM RECENT EVENTS

- When distance less than 100

IBM RECENT EVENTS

WOVKI LINK-

https://wokwi.com/projects/347841927477985876